# Remotely sensing river greenhouse gas exchange velocity using the SWOT satellite

Craig B Brinkerhoff1,✉, Colin J Gleason1, Christopher J Zappa2, Peter A Raymond3, and Merritt E Harlan1

1 Department of Civil & Environmental Engineering, University of Massachusetts, Amherst, MA
2 Lamont-Doherty Earth Observatory, Columbia University, Palisades, NY
3 School of the Environment, Yale University, New Haven, CT

✉ Correspondence: Craig B Brinkerhoff <cbrinkerhoff@umass.edu>

## Key Points

- BIKER (Bayesian Inference of the K600 Evasion Rate) algorithm predicts gas exchange velocity from simulated SWOT data without calibration

- BIKER is marginally influenced by expected SWOT measurement errors

- BIKER and near-daily SWOT data will allow for the study of gas exchange spatiotemporal dynamics at novel temporal resolutions

## Keywords

gas exchange, fluvial geomorphology, remote sensing, open-channel flow, SWOT, biogeochemistry

## Abstract

Two decades of research has shown that the global river network emits significant amounts of greenhouse gas. Despite much progress, there is still large uncertainty in the temporal dynamics of gas exchange and thus carbon emissions to the atmosphere. Much of this uncertainty stems from a lack of existing tools for studying the spatiotemporal dynamics of gas exchange velocity k600 (the rate of this diffusive transport). We propose that the NASA/CNES/UKSA/CSA Surface Water and Ocean Topography (SWOT) satellite can provide new insights to fluvial gas exchange modeling upon launch and subsequent data collection in 2022. Here, we exploit the distinct geomorphology of SWOT-observable rivers (> 50m wide) to develop a physical model of gas exchange that is remotely sensible and explains 50% of log-transformed variation across 166 field measurements of k600. We then couple this model with established inversion techniques to develop BIKER, the ‘Bayesian Inference of the k600 Exchange Rate’ algorithm. We validate BIKER on 47 SWOT-simulated rivers without an in-situ calibration, yielding an algorithm that predicts the k600 timeseries solely from SWOT observations with a by-river median Kling-Gupta Efficiency of 0.21. BIKER is better at inferring the temporal variation of gas exchange (median correlation coefficient of 0.91), than reproducing the absolute rates of exchange (median normalized RMSE of 51%). Finally, BIKER is robust to measurement errors implicit in the SWOT data. We suggest that BIKER will be useful in mapping global-scale fluvial gas exchange and improving CO2 emissions estimates when coupled with river CO2 models.

## 1 Introduction

Rivers play a critical role in the budgeting and accounting of the global carbon cycle under climate change. Following Cole et al. (2007), the global river network is recognized to emit substantial amounts of carbon to the atmosphere via evasion (gas exchange driven by diffusion and near-surface turbulence at the air/water interface), in addition to their long understood role in transporting carbon to the oceans via downstream advection. Current estimates of total carbon dioxide evasion (FCO2) to the atmosphere from the global river network vary from 1800-2000 Tg C/yr (Liu et al., 2022; Raymond et al., 2013), with 650 Tg-C/yr and 167 Tg-C/yr coming from mid-to-large rivers (Lauerwald et al., 2015) and mountain streams (Horgby et al., 2019), respectively. Despite rivers’ small percentage of the global land surface (0.67%- Liu et al., 2022), this carbon flux is on par with the total oceanic CO2 uptake rate (2600 Tg-C/yr- Gruber et al., 2019; Horgby et al., 2019) and the global forest carbon uptake rate (2400 Tg-C/yr- Pan et al., 2011).

River CO2 evasion is increasingly better constrained and is clearly a critical component of the global carbon cycle. Equation 1 represents this riverine flux given ΔCO2 (CO2water−CO2air) and the gas exchange velocity k. Consult Appendix A for notation used throughout this study.

FCO2=kΔCO21

There is a robust existing literature exploring spatiotemporal patterns in ΔCO2water (e.g. Aho et al., 2021; Aho & Raymond, 2019; Crawford et al., 2017; Liu & Raymond, 2018; Peter et al., 2014; Ran et al., 2017; Raymond et al., 2000; Rocher‐Ros et al., 2019). This work has lead to recent river-reach explicit modeling of ΔCO2 using global hydrography datasets at up to monthly temporal resolutions (Brinkerhoff et al., 2021; Horgby et al., 2019; Liu et al., 2022; Saccardi & Winnick, 2021), but an equivalently sophisticated representation of kCO2 is still lacking.

The structure of Equation 1 necessitates that calculations of FCO2 are highly sensitive to estimates of k. However, k can only be estimated via a known gas flux, eddy-covariance measurements, or tracer additions to the stream (Hall & Ulseth, 2020). In trying to constrain the global fluvial FCO2 flux across millions of rivers, this calculation is impossible and necessitates the use of predictive models for k that are based on easily obtained river hydraulic properties. In that vein, over 20 empirical models exist to predict k from river hydraulics, generally using some combination of mean velocity U, shear velocity U*, width W, depth H, and slope S as predictors (Hall & Ulseth, 2020; Wang et al., 2021). These models usually predict k600, or k normalized by a Schmidt number (Sc) of 600 to remove the effect of water temperature and gas type from predictive models, as warmer waters and lower Sc numbers each increase gas exchange rates (Hall & Ulseth, 2020). Specifically, k600 reflects the kCO2 at 20 degrees Celsius. Through this normalization, these models focus solely on physical explanations for variation in k (Hall & Ulseth, 2020).

Applying these k models across watersheds, regions, or continents is called ‘upscaling.’ This upscaling allows for quantifying the difficult-to-estimate k term in Equation 1 for any arbitrary number of rivers, but also changes the base parameters that ultimately control the final estimate of FCO2. That is, by making k a function of hydraulics, FCO2upscaled is now a direct function of river hydraulics. This functional relationship is described in equation 2. It suggests that global FCO2 estimates are not only at the mercy of the accuracy and spatiotemporal resolution of ΔCO2, but also the accuracy and resolution of our river hydraulics estimates.

FCO2upscaled=fkCO2,ΔCO2=fU,H,S,W,ΔCO22

Global upscaling has been performed using various techniques. Raymond et al. (2013), Lauerwald et al. (2015), and Horgby et al. (2019) all relied on kCO2 values indirectly estimated using mean annual streamflow models and hydraulic scaling equations to predict the hydraulic terms used to in turn predict kCO2, while Borges et al. (2015) used a combination of the above method and a constant kCO2 in space and time to upscale over Africa. More recently, Liu et al. (2022) performed a first upscaling assessment of monthly temporal dynamics in global river FCO2, though they relied on monthly modeled streamflow and used the same model for k600 as previous studies (Raymond et al., 2013). In all of these foundational studies, the temporal dynamics of kCO2 (and thus dynamics in FCO2) were ignored because of hydraulic data limitations. It has been shown at the field-scale that temporal dynamics of gas exchange can vary widely from site to site (Wallin et al., 2011), but it has remained impractical to obtain temporally explicit kCO2 at continental-to-global scales.

Wang et al. (2021) recently attempted to address this global k problem by simulating the gas exchange velocity of dissolved oxygen (kO2) in 35 rivers of many sizes (widths ranging from 0.23-349m) using a stream metabolism inverse model (Appling et al., 2018) and in situ dissolved oxygen datasets to infer what kO2 must have been to produce their ‘observations’. They then compared this simulated dataset against direct estimates of k, finding similar performance and parameter values for process-based models of gas evasion. However, they were still limited by a lack of direct hydraulic measurements and had to rely on hydraulic scaling equations to estimate river depth and velocity. Even though approaches like Wang et al. (2021)’s are incredibly useful for expanding our mechanistic understanding of gas exchange, they are less useful for global upscaling purposes as they rely on high fidelity in situ dissolved oxygen data for every river (Hall & Ulseth, 2020).

We have established that literature has a reasonably good understanding of ΔCO2 and a relatively poorer understanding of k600 (and therefore kCO2) across large areas and in time. In theory, the discrepancy between the quality of our ΔCO2water and kCO2 estimates could be alleviated if direct hydraulics measurements (and in turn kCO2 via Equation 2) were available at the global scale at a sufficient temporal resolution. Spatially and temporally dynamic hydraulic measurements in turn would also address the uncertainty regarding continental-to-global scale temporal dynamics of gas exchange noted earlier.

Conveniently, these hydraulic data will soon be available via the upcoming NASA/CNES/UKSA/CSA Surface Water and Ocean Topography (SWOT) satellite mission. SWOT is expected to launch in late 2022 and provide the world’s first direct measurements of global water surface extent and elevation (and therefore water surface slope) at novel temporal resolutions. SWOT is a wide swath radar interferometer and will sample rivers every 1 to 7 days per 21 day repeat cycle, measuring rivers wider than 100m with a goal of expanding to rivers at least 50m wide (Biancamaria et al., 2016). Via its direct hydraulic measurements, SWOT is expected to usher in a sea change in global-scale hydrology, and could similarly influence fluvial biogeochemsitry if techniques are developed to ingest SWOT data and infer k. In this context, we borrow tools from fluvial geomorphology and existing SWOT algorithms to begin addressing the current knowledge gap in the spatiotemporal dynamics of gas exchange velocity. More specifically, we seek to answer the following two questions:

- What is the performance of a physically-based hydraulic model for k600 unique to SWOT-observed rivers?

- How well can we exploit such a model to infer k600 (and its uncertainty) solely from SWOT observations?

To answer the first question, we use hydraulic geometry- the fundamental geomorphic relationships between streamflow and channel shape (Gleason, 2015; Leopold & Maddock, 1953) to develop a process-based model for large-river k600 (here defined as wider than 50m to align with SWOT). We then take these findings and explore the second question by implementing this hydraulic model, which defines k600, within an algorithm named BIKER (‘Bayesian Inference of the k600 Evasion Rate’) to infer k600 solely from SWOT measurements. The goal of BIKER is to require no in situ inputs of any kind (although in situ data could be ingested and would improve results) such that it is globally implementable on any SWOT-observable river. We validate BIKER on 47 SWOT-simulated rivers (as SWOT has not yet launched) and explore BIKER’s robustness to the expected measurement errors implicit in the satellite’s observations. Finally, we also couple BIKER’s k600 estimates with ΔCO2 to predict gas fluxes and compare these against established literature methods that rely on hydraulic scaling equations.

This paper is split into two distinct parts: gas-exchange theory/model development (Section 2) and BIKER algorithm development/validation (Section 3). Section 3 is fundamentally dependent on the results presented in Section 2, therefore Section 2 presents both theory and results. Both sections detail the data used. We conclude with a discussion (Section 4) that encompasses both gas exchange theory and remote sensing. Figure 1 conceptually maps out the algorithm’s approach to inferring k600 from SWOT data, while Figure S1 details the entire study’s workflow.

Figure 1: Conceptual overview of the BIKER algorithm. 1: SWOT will directly measure water surface width and elevation (and thus slope) at a series of cross-sections within mass-conserved river reaches. 2: These hydraulics measurements are used to calculate turbulent dissipation in the river channel (Section 3). 3: Turbulent dissipation is used to infer k600 via a process-based model (Section 2).

## 2 Predicting k600 from large-river hydraulic geometry

### 2.1 Data

We develop our hydraulic model for k600 using multiple datasets of field-estimated k600 and measured stream hydraulics collected from the literature. In total, we obtain 763 estimates of k600, with 701 of these estimates previously gathered by Raymond et al. (2012) and Ulseth et al. (2019). The remaining estimates come from Churchill et al. (1964) and Owens et al. (1964). See Table S1 for a complete list of the studies that collected these data. All k600 estimates come from tracer studies and thus define k600 at the reach scale.

The 763 estimates cover different times of year and hydrological events. They include both individual estimates and repeat estimates in over 500 river reaches across the United States, Wales, Switzerland, and Austria. They span a wide variety of environments from temperate higher-order rivers to small mountain streams and represent a full range of river flows (width ranges from 0.26m to 1,742m, discharge ranges from 8e-4 m3/s to 489 m3/s, and k600 ranges from 0.1 m/dy to 4,118 m/dy). While there are still geographic and hydrologic biases in this dataset, it is to our knowledge the largest such dataset of field-estimated, reach-scale k600 where hydraulic data is concurrently available.

In addition to hydraulics measured alongside k600 and reported above, we expand our dataset of stream hydraulics using a previously published compilation of in situ hydraulic measurements (Brinkerhoff et al., 2019). That dataset contains over 530,000 unique measurements of river channel velocity, width, and discharge from across the continental United States, originally made to calibrate United States Geological Survey (USGS) streamgauge rating curves and made public by the USGS. This dataset is used to calculate how frequently SWOT observable rivers meet our large-river hydraulic assumptions (Section 2.3).

### 2.2 Process-based hydraulic modeling of river k

k scales with near-surface turbulence in turbulent streamflows (Hall & Ulseth, 2020), and extensive field and laboratory experiments have converged on the ‘small-eddy model’ for k as derived by Lamont & Scott (1970) and empirically anticipated by Calderbank & Moo-Young (1961). This model scales k via the smallest-scale turbulent eddies and has been repeatedly empirically validated in freshwater systems (e.g. Katul et al., 2018; Lorke & Peeters, 2006; Moog & Jirka, 1999b; Tokoro et al., 2008; Vachon et al., 2010; Wang et al., 2021; Zappa et al., 2003, 2007). The small-eddy model is provided as Equation 3, where ϵ is the dissipation rate of near-surface turbulence, v is the kinematic viscosity, and Sc is the Schmidt number.

k∝Sc−1/2vϵ1/43

Some laboratory and field observations additionally suggest that open channel flows with small bed roughness do not exhibit homogeneous surface dissipation across the entire reach’s air-water interface (Moog & Jirka, 1999a; Talke et al., 2013). Given this observation, Moog & Jirka (1999a) proposed an extension to the small-eddy model, additionally scaling k600 using a shear Reynold’s number Re* formulation (equation 4, referred to as the ‘Reynolds extension’ model). The Reynolds extension model is hypothetically useful in low-turbulence scenarios where a relative lack of large-scale eddies effectively ‘filter out’ the number of small-eddies that actually reach the interface and initiate gas exchange (Talke et al., 2013). While scaling k via a shear Reynold’s formulation is sometimes done to parameterize wave-breaking gas exchange models in the open ocean (e.g. Brumer et al., 2017; Zhao et al., 2003; Zhao & Toba, 2001), it is infrequently done in rivers. In the context of BIKER, we chose to test this model because large, SWOT-observable rivers are generally the smoothest, least-turbulent flows along the stream-to-ocean continuum where small eddies might not reach the surface. Further, to our knowledge, the Reynolds extension model has never been empirically tested in predicting river k, aside from confirming that large-scale eddies differentially move turbulence to the surface in a large river (Talke et al., 2013).

k∝Sc−1/2vϵ1/4Re*3/84

Equations 3 and 4 both rely on ϵ, which is difficult to measure. When working at large scales, a commonly used ϵ model assumes that all turbulence is generated at the bed and transported to the air-water interface via the log-law-of-the-wall (Equation 5). Another approach specific to fluvial settings models k via ‘form-drag dissipation’ (Equation 6) which is equivalently the total stream power per unit mass water. This normalized stream power captures the bulk frictional resistance (and thus energy dissipation) via channel banks, meanders, bars, etc. that is unique to rivers (Moog & Jirka, 1999b). Authors have since shown that Equation 6 can reasonably predict k in rivers and streams (Raymond et al., 2012; Ulseth et al., 2019; Wang et al., 2021).

ϵ=ϵS=U*3H5

ϵ=ϵD=gSU6

### 2.3 Deriving a large-river k600 model

Given the theoretical context provided in Section 2.2, we now turn to SWOT-observable rivers specifically. Rivers and streams change predictably along their longitudinal profile from source to sea, and we can exploit the hydraulic geometry of large rivers at the end of this continuum to estimate k in SWOT rivers. In general, as river size increases, channels become more rectangular, their shapes elongate (becoming wider quicker than they become deeper) and their hydraulic radii begin to approximate their mean flow depth, i.e. Rh≈H (Leopold & Maddock, 1953), a common assumption in hydraulic and geomorphic modeling of large rivers. For example, SWOT-observable river flows have an average RhH ratio of 0.98 and standard deviation of only 0.02 (n = 22452; see Text S1 for how we built this dataset). We refer to these rivers as ‘hydraulically-wide’.

We therefore assume that all SWOT-observable rivers are hydraulically-wide to derive a model for gas exchange. The overall goal is to reduce the equations down to their fundamental parameters, identifying which terms are SWOT observable and limiting the number of terms not directly measurable via SWOT. We first impose ϵ=ϵD and RH=H on the Reynolds extension model (Equation 4) and then log-transform to maintain homogeneous variance across the orders of magnitude of k600 to arrive at Equation 7 (with statistical coefficient β1). Equation 7 thus defines gas exchange velocity solely as a function of slope, mean flow depth, and mean flow velocity and is theoretically valid only in a hydraulically-wide channel.

logk600=β1+716loggS+14logU+916logH7

We also test the performance of three other models for predicting k in hydraulically-wide channels via the other three unique combinations of Equations 3-4 and Equations 5-6. While the complete model derivations and results for all four models are provided in Text S2 and Figure S2, the final and best-performing model (Equation 7) is presented and used below.

### 2.4 Model validation

With Equation 7 derived, we now test its strength of fit in hydraulically-wide river flows. We validate on the dataset of in situ estimates of k600, after filtering for those made in hydraulically-wide channels, which was defined as flows whose hydraulic radius was within 1% of their mean flow depth. All told, there are 166 estimates of hydraulically-wide k600. Equation 7 is assessed via a Bayesian linear regression model, where the theoretically-derived coefficients from equation 7 are used as reasonably informative priors for the regression coefficients. The intercept and model uncertainty both use uninformed priors. See Text S3 for the model specification. We assess the goodness-of-fit using the posterior mean coefficient of determination (r2) following (Gelman et al., 2019).

Figure 2: Empirical testing of our large-river k600 model on 166 measurements made in hydraulically-wide rivers. Dashed grey line denotes the 1:1 line. For the model, points reflect posterior means while lines reflect the 95% credible intervals.

Equation 7 explains 50% of variation in observed (log-transformed) k600 in hydraulically-wide rivers and accurately captures the scaling dynamics of observed k600 as well (Figure 2). The posterior mean in natural log space for β1 is 3.89 (Table 1). While the individual residuals can be quite large (indicating that additional processes and/or field estimation error are presumably controlling the 50% unexplained variation in log-transformed k600), the general scaling of k600 with river hydraulics is strongly captured. We also tested two non-Bayes implementations of equation 7 using 1) the theoretically-derived coefficients and 2) a fully empirical line fitting approach (Table 1). Interestingly, the data exerted little-to-no influence on the theoretically-derived priors, and both the empirical and posterior coefficients were virtually identical to those obtained theoretically (Table 1), indicating that equation 7 is a good theoretical model for gas exchange in hydraulically wide flows.

Table 1: Comparison of three regression models for k600 model for hydraulically-wide rivers. ‘Theoretical’ refers to the model coefficients derived by imposing a hydraulically wide channel on the ‘Reynolds-extension’ model for gas exchange (section 2.3). ‘Empirical’ refers to the result of fitting the coefficients by least-squares regression. ‘Bayesian’ refers to the posterior means of the coefficients, conditional on priors informed by the ‘Theoretical’ coefficients (Text S3, Figure 2).

| Equation 7 results | ‘Theoretical’ | ‘Empirical’ | ‘Bayesian’ |
| --- | --- | --- | --- |
| α1 | 716≈0.44 | 0.42 | 0.43 |
| α2 | 14≈0.25 | 0.32 | 0.31 |
| α3 | 916≈0.56 | 0.5 | 0.52 |
| β1 (natural log space) | 3.85 | 3.85 | 3.89 |
| r2 | 0.5 | 0.5 | 0.5 |

Compared to other combinations of Equations 3-6 (Figure S2), there is also a better fit using equation 7 thanks to the addition of the Reynold’s number scaling for low-turbulent flows (as expected). The success of the final model in hydraulically-wide channels (provided as equation 8 using the posterior means) provides us with a physical-model for gas evasion built with SWOT in mind. The river hydraulics terms in Equation 8 (U, H, and S) can either be directly measured or reasonably inferred from SWOT measurements, effectively opening the door for remotely sensing the gas exchange velocity.

logk600=3.89+0.43loggS+0.31logU+0.52logH8

## 3 BIKER algorithm development and validation

### 3.1 Data

To validate BIKER, we cannot use actual SWOT measurements as SWOT has yet to launch. In the hydrology literature, it has become standard practice to benchmark SWOT-related algorithms on “SWOT-like” data (Durand et al., 2016; Frasson et al., 2021). We use 47 SWOT-simulated rivers for validation, where these simulated rivers are simply reach-averaged hydraulic model outputs where the water surface heights, slopes, and widths are labelled as RS observations and are used as the sole inputs to BIKER (Figure S1). These datasets are created using standard hydraulic models forced with known inflows and measured bathymetry to model the hydraulic response of the rivers, and then those terms visible to SWOT are extracted to produce hydraulically realistic synthetic observations. These data were published by Frasson et al. (2021) and Durand et al. (2016). See Figure S3 for a map of these river’s locations along the global SWOT river network (Altenau et al., 2021).

There is considerable geographic bias in our validation rivers, with rivers only present in North America, Western Europe, and Bangladesh. Further, no Arctic rivers are included. We acknowledge that this bias limits our ability to validate BIKER across many environments ahead of SWOT’s launch. However, it is a sufficient validation set for a first proof-of-concept study consistent with the hydrology literature for SWOT. Further, the data requirements to create these test cases are strict and the processing time is enormous. Also note that SWOT water surface slope measurements will have a lower detection limit of 1.7 cm/km (Biancamaria et al., 2016), and therefore any slope measurement in our data less than this threshold was reassigned this minimum value.

We validate BIKER under two different ‘error scenarios’ (section 3.2.2). While SWOT will provide river surface measurements of novel quality and resolution, as with all remote sensing products there are expected errors that will be implicit in these measurements. Here, we validate BIKER under a ‘no-measurement-error’ scenario that reflects an unrealistic measurement as if SWOT has perfect accuracy and precision: we use the hydraulic model of 47 rivers directly as a first test of algorithm validity. 16 of these rivers are then additionally validated under a ‘measurement-error’ scenario that more closely mimics expected SWOT by adding realistic radar errors and sampling along the satellite’s future ground track. SWOT river error modeling was developed by Durand et al. (2020) and then applied to these 16 rivers by Frasson et al. (2021). This error modelling is non-trivial and computationally expensive, and so Frasson et al. (2021) were limited to only 16 test cases with SWOT measurement errors. Likewise, we stick to these 16 rivers for the same reason. These rivers are detailed in Figure S3. Given that it is only 16 synthetic rivers, caution should be used in over generalizing our results beyond our proof of concept.

For CO2 evasion and carbon emissions calculations, we use 26 bi-weekly dissolved CO2 samples collected by Beaulieu et al. (2012) at one location in the Ohio River (Figure S3) for one calender year from 2008-2009 (Figure S4). Note that this CO2 data is for the Ohio River only but was applied to all rivers to provide a physically realistic signal for CO2 fluxes with meaningful seasonality and dynamics. Therefore, the raw carbon emissions estimates presented in this paper are meaningless in the context of actually measured carbon emissions from these rivers but are better than specifying CO2 concentrations devoid of context. These data are necessary as we are interested in the effect of BIKER k600 errors on eventual fluxes and comparing these fluxes with published methods. Therefore, applying these ‘unit’ CO2 values allows for such a comparison by providing a realistic timeseries.

### 3.2 Section 3 methods

#### 3.2.1 BIKER

To develop BIKER, we follow the work of Hagemann et al. (2017), Brinkerhoff et al. (2020), and Durand et al. (2014) to infer k600, H, and U from SWOT observations. These Bayesian approaches to inverse modeling via SWOT data start from Bayes rule (Equation 9), where Θ is some set of non-remotely-sensible parameters we want to solve for (including k600), x is the observed data, fx|Θ is the sampling model where data are conditional on the parameters, and pΘ is the joint prior distribution of the parameters. Therefore, we are interested in solving for pΘ|x, or the ‘posterior’ distribution. Note that px is usually computationally intractable to integrate exactly, but Bayesian inference requires only the proportionality to be specified: pΘ|x∝fx|ΘpΘ. Sampling algorithms are then used to approximate the actual posterior distribution, as is done in BIKER.

pΘ|x=fx|ΘpΘpx9

The heart of BIKER is its reformulation of the k600 model (Equation 8) as a Bayesian sampling model that is conditional on the non-remotely-sensed parameters (i.e. fx|Θ). This approach is similar to the ‘McFLI’ (Mass-Conserved Flow Law Inversion) logic used in some SWOT remote sensing of discharge algorithms (Gleason et al., 2017). To start, we write k600 as a function of SWOT-observables W and S. This algebra is carried out using Equation 8 and Manning’s equation for mean flow velocity (U=1nRh2/3S1/2). Following Section 2.3, we continue to assume that the channel is hydraulically-wide (Rh=H=AW). To leverage additional SWOT data, we use the “Durand transform” originally published by (Durand et al., 2014): the wetted channel area A is further split into the SWOT-unobservable portion A0 and SWOT-observable portion dA where dAit=t′:Wt′≤Wt​Wt′δHet′ for cross-section i and timestep t within a mass-conserved river reach.

All of the above algebra simplifies to Equation 10. Conveniently, k600 as estimated by tracer additions to a stream is inherently a reach-scale quantity (in a mass-conserved reach). Therefore, Equations 7, 8, and 10 all yield a reach-scale k600 (i.e. k600i=k600∀i), thus lowering the number of parameters BIKER must infer and making the problem much better constrained.

logk600t=3.89+0.4320logg+0.5862logSi,t+0.3084log1ni+0.7282logA0i+dAi,tWi,t10

Next, Equation 10 is re-written as a Bayesian sampling model fx|Θ, in which the joint data distribution x (i.e. SWOT observations) is sampled from the joint parameter distribution (Θ). We first rearrange equation 10 to isolate x from Θ (equation 11). Then, equation 12a-c re-expresses equation 11 as a normal distribution with standard deviation σk600. The uncertainty expressed by σk600 arises from uncertainties in 1) parameters in Equation 8, 2) Manning’s equation, and 3) the hydraulically wide channel assumption.

3.89+0.4320logg+0.5862logSi,t−0.7282logWi,t=logK600t+0.3084logni−0.7282logA0i+dAi,t11

x=3.89+0.4320logg+0.5862logSi,t−0.7282logWi,t12a

Θ=logK600t+0.3084logni−0.7282logA0i+dAi,t12b

x NΘ,σk600212c

Equations 9 and 12 necessitate that we specify prior distributions for the parameters A0i, k600t, and ni. Prior distributions formalize the a priori estimates and uncertainties for the non-remotely-sensed terms. More intuitively, BIKER priors represent our ‘prior river knowledge’ of what A0i, k600t, and ni probably are for some river since they cannot be directly remotely sensed. This method of hyperparameter assignment is analogous to the ‘empirical Bayes approach’ to Bayesian inference (Hoff, 2009). Our goal in prior specification was to rely on absolutely no in situ information such that we could run this method on any river on Earth solely using SWOT observations. In theory, more informed priors via various a priori information about a specific river will improve BIKER performance, but here we chose to test the fully generalized algorithm. Therefore, the validation presented here is a ‘worst-case scenario’, wherein BIKER performance should improve with better prior information on the river. In that context, we used a variation of the prior specification method developed by Brinkerhoff et al. (2020), who developed ‘geomorphic river types’ with distinct prior sets for A0i and ni. These priors are assigned to a river solely using SWOT data W and S, therefore meeting our needs for complete global implementability. Prior assignment for k600t was developed similarly (all prior specifications are elaborated on in Text S4).

With the sampling model described (fx|Θ = Equation 12a-c) and priors pΘ specified (Text S4), a joint posterior distribution conditional on the SWOT observations (pΘ|x∝fx|ΘpΘ) is therefore also specified. To approximate this distribution, we use a Markov Chain Monte Carlo (MCMC) algorithm implemented using the Stan probabilistic programming language. Specifically, Stan uses a Hamiltonian Monte Carlo sampler which reduces computation time relative to other sampling algorithms (Hagemann et al., 2017).

#### 3.2.2 BIKER validation

BIKER provides a timeseries of k600: for each SWOT observation, it yields a unique k600. There are, to our knowledge, no datasets of k600 over time approaching the temporal density of our simulated SWOT rivers. We therefore apply Equation 8 as validated in Figure 2 to specify k600 given the true hydraulics of each case and compare BIKER’s inversion to that value: given observed hydraulics, ‘observed’ k600 comes from Equation 8. Remember that SWOT cannot observe below the water surface and therefore cannot measure U or H (hence the need for Equation 10), and that all SWOT observations contain errors in both space and time (hence Equation 12). We acknowledge that there is error in Equation 8 as shown in Figure 2, but this error can be explicitly parameterized in our Bayesian system (elaborated on in Text S5). Therefore, the BIKER validation presented here is an exercise to see how well the imperfect and partial SWOT observations can infer k600 given the hydraulic assumptions in Equation 10 and uncertainty in the data itself. Note also that we have already validated Equation 7 in Figure 2 and Table 1.

We validate BIKER as a timeseries of k600 for each river using the BIKER posterior means. Our error metrics consider the timeseries nature of the problem and are formally defined in Table S2. They consist of the correlation coefficient r to quantify accuracy of BIKER’s temporal dynamics, the root mean square error normalized by the observed mean (NRMSE) and normalized mean absolute residual error (NMAE) to assess bias, and the Kling-Gupta Efficiency (KGE). KGE is frequently used to assess streamflow prediction and simultaneously assesses accuracy in both bias and dynamics. While a value greater than -0.41 means the model outperforms a uniform prediction of the mean (Knoben et al., 2019), generally KGE scores are interpreted as being meaningful in ungauged settings if > 0.

#### 3.2.3 Carbon emissions validation

It is one thing to accurately model the temporal dynamics of k600 as above, but researchers are often most interested in the actual carbon emitted from river to atmosphere. Per Equation 2, emissions upscaling is done using river hydraulic models to estimate kCO2 and in turn FCO2upscaled. However, streamflow data and/or model outputs are more readily modeled at the global scale than river channel geometry, and so upscaling models usually predict U and H as functions of streamflow (Q) using hydraulic geometry scaling relationships. This workflow effectively reduces Equation 2 to Equation 13. It is worth stressing that these literature upscaling workflows rely on in situ streamflow records and/or high-quality streamflow models.

FCO2upscaled=fkCO2,ΔCO2=fQ,ΔCO213

Conversely, BIKER represents a new way of approaching Equation 13 compared to existing literature models: BIKER has no reliance on a streamflow model nor hydraulic geometry scaling relationships and only requires that a river is SWOT-observable. We are therefore interested in how the final carbon emissions that result from BIKER compare against literature methods that use Equation 13. We have the data to test four different models for fluxes: ‘BIKER’, ‘Raymond 2013’, ‘Raymond 2012’, and ‘Brinkerhoff 2019’. These latter three approaches all use the same philosophy for k600: making hydraulic and geomorphic assumptions to associate k600 with observed hydraulics before using the CO2 data as a realistic timeseries to yield fluxes per Equation 13. In all three approaches, these observed hydraulics are streamflow, while BIKER uses only SWOT observations. Therefore, the advantage of BIKER is in its ease of application, as SWOT will observe all global rivers wider than 50m while streamflow observations are extremely geographically limited. But, BIKER is only attractive if it can produce fluxes with similar errors to published methods. Text S6 and Table S3 fully describe these three literature models.

To benchmark BIKER against these literature methods, we pair the 26 biweekly CO2 and water temperature samples from Beaulieu et al. (2012) (Section 3.1, Figure S4) with every 14th set of daily SWOT observations (as the CO2 data is bi-weekly). We then calculate FCO2 using Equation 1, an atmospheric CO2air of 400 μatm, and a Sc estimated following Raymond et al. (2012). The k in Equation 1 is obtained using BIKER or the three literature models (Table S3 and Text S6). Finally, we estimate a pseudo yearly total carbon emission rate (via CO2 evasion) by applying each river’s mean FCO2 over the river’s surface area and summing all rates across rivers, remembering that we are applying ‘unit’ CO2 data to all rivers.

### 3.3 Section 3 results

#### 3.3.1 BIKER

In general, the temporal dynamics of k600 are reproduced quite well by BIKER (Figure 3). In the best performing rivers, both bias and temporal dynamics are strongly captured (Figure 3e-f). Notably, there is sometimes positive bias in the estimates (e.g. the Connecticut and Iowa rivers, Figure 3c-d). Some rivers yield the correct temporal dynamics, but the magnitude of these values is stretched relative to the observed (e.g. Ohio River and Seine River, Figure 3a-b). In these two examples, temporal trends are still reasonably inferred even though the magnitude of the estimates is quite wrong. The size of posterior uncertainty in k600 does not appear to be associated with overall algorithm performance, with both certain and uncertain results spread across the rivers, regardless of their KGE (Figure 3, Figure S5).

Figure 3. Representative (6 out of 47) river timeseries plots of k600. Orange is observed, while green is BIKER posterior means and uses SWOT measurements as its sole input. The green ribbon indicates the 95% credible intervals for the BIKER posteriors. Rivers are sampled from the three tertiles of KGE scores. (a-b) are poorest performing rivers, (c-d) are in the middle, and (e-f) are the best performing rivers. Y axis is normalized by maximum observed values to compare visually. Consult Figure S5 for all 47 timeseries plots (assuming no measurement error) and Figure S6 for the 16 rivers with measurement errors.

Overall, river performance across error metrics (Table S2) is reasonable given the strict validation setup we have employed (Figure 4). Median river KGE is 0.21 and median river r is 0.91 (Figure 4). Further, 31/47 rivers outperform a uniform prediction of the mean (KGE = -0.41). The correlation coefficient r out-performs the other metrics which assess bias (Figure 4). This result indicates strong inference of each river’s temporal k600 dynamics given that absolutely no in situ information is being used to predict k600. NRMSE has a median score of 0.51 (Figure 4), highlighting many rivers which have notable positive biases (Figure 3 also confirms this result visually). Median NMAE is 47% (Figure 4). Taken in aggregate, Figures 3-4 indicate that BIKER is quite good at capturing temporal dynamics in k600, however there is often positive bias in its estimates.

Figure 4. Performance metrics across all 47 rivers, ploted as empirical cumulative density functions (eCDFs). Each subpanel is labelled by its performance metric (defined in Table S2). Dashed lines denote median scores.

BIKER is robust to the measurement errors that will be implicit in SWOT’s observations of river width and slope (Figure 5). BIKER performs nearly identically across all four error metrics, particularity with respect to the metrics that singularly assess bias or correlation errors (Figure 5b-d). The three very poor KGE rivers actually improve under the error scenario (Figure 5a), though this is likely an artifact of BIKER already not working in these rivers. Given these results, we deem that SWOT measurement error does not exert a significant influence on BIKER performance and so the results presented for the rest of the manuscript assume no measurement error in order to use all 47 rivers.

Figure 5: Comparison of BIKER performance when considering measurement error in the SWOT observations for 16 rivers. Each subpanel is labelled by its performance metric (Table S2). Rivers that fall within the purple zone get worse when accounting for measurement error, while rivers in the green get better. Note that some axes are flipped in order to visualize the ‘best performances’ in the upper-right-corner of each sub-plot. Black line denotes linear regression to aid in visualization.

Finally, errors/biases associated with the prior k600 are correctly propagated through the posterior in an approximately 1:1 manner (which is expected), except for a subset of rivers in which posterior error actually increases relative to the prior (Figure 6a). We explore why this phenomenon happens below.

Figure 6: Empirical cumulative density functions for prior and posterior k600 NMAE. See Table S2 for metric definitions. a) all rivers. b) Only rivers with a coefficient of variation (CV) of observed k600 > 0.10. c) Only rivers with CV > 0.20. d) Only rivers with CV > 0.30. Overall we see that rivers with greater temporal variation in k600 behave better for BIKER, i.e. Bayesian inference reduces bias from prior to posterior.

Recall that BIKER relies on a timeseries of SWOT data, and that these timeseries may not be representative of the full spectrum of k600 values that are actually experienced in the river, therefore potentially biasing both the prior estimation methods (Text S4) and the actual Bayesian inference. Put another way, we suggest that if a SWOT timeseries does not sufficiently capture a river’s temporal dynamics, it will introduce additional error to the inference results. To test this hypothesis, we subset our validation dataset by progressively higher coefficients of variation (CV) (> 10%, >20%, and >30%). Posterior bias drops once the temporal variability of the SWOT data is sufficiently high (Figure 6b-d), with BIKER posterior error the smallest, and much less than prior error, in Figure 6d. This result is elaborated on in Section 4.2.

#### 3.3.2 Carbon Emissions

Finally, we carry these k600 estimates all the way to annual carbon emissions rates and compare BIKER against three established in situ techniques in the literature. It is important to remember that only BIKER is completely globally implementable, while the other three models necessarily rely on having a streamflow record or high-quality routed streamflow readily available. Therefore, the in situ methods reflect their ‘best-case scenarios’ while BIKER reflects its worst case scenario, where priors are set entirely from SWOT observations and are generally the least informative they will ever be. This validation setup means that BIKER’s annual carbon emission estimate can only improve from what is presented here. We also stress again that the raw emissions rates here are meaningless as they are calculated from an identical CO2water timeseries applied to every river. We are principally interested in relative differences between techniques employed.

Figure 7 compares the annual carbon emissions rate (via FCO2) from the rivers using BIKER posterior means and the three stream gauge-based HG models. BIKER outperforms the gauge-based approaches, nearly correctly inferring the annual carbon emissions rate (7.87 Tg-C/yr for BIKER versus 6.9 Tg-C/yr observed). The three HG models overestimate this emissions rate: 11.11 Tg-C/yr, 9.32 Tg-C/yr, and 12.22 Tg-C/yr for ‘Raymond 2013’, ‘Raymond 2012’, and ‘Brinkerhoff 2019’ respectively. ‘Raymond 2012’ falls within the BIKER credible intervals and is reasonably close to the observed value, while ‘Raymond 2013’ overestimates the emission rate. ‘Brinkerhoff 2019’‘s lower confidence interval is comparable to BIKER’s higher credible interval. BIKER’s relatively stronger performance than the in situ models is elaborated on in Section 4.3. Finally, BIKER’s uncertainty is on par with the in situ technique (‘Brinkerhoff 2019’), despite being obtained solely from SWOT data. Taken in aggregate, BIKER provides a strong upscaling estimate of the annual carbon emission rate for the rivers and is either similar or better than established in situ techniques (Figure 7).

Figure 7: Yearly carbon emissions rate via CO2 evasion across all rivers. Completely remotely-sensed methods are colored in red (with 95% credible intervals), in situ methods in purple (with 95% confidence intervals when available), and the observed in orange. Consult section 3.2.3 for details on the ‘observed’ flux.

## 4 Discussion

In this paper, we propose that the soon-to-launch SWOT satellite will provide enough hydraulic measurements to analyze the temporal dynamics of k600, and therefore allow for a global-scale analysis of spatiotemporal trends in large-river k600 once SWOT launches. In preparation for SWOT’s launch, we developed 1) a wide-river-specific hydraulic model for k600 that explains 50% of variation in k600 and 2) the BIKER algorithm to infer k600 using no on-the-ground information. Validating on 47 SWOT-simulated rivers, we show strong recovery of rivers’ temporal k600 dynamics and a hypothetical total annual carbon emission rate across all 47 rivers (section 3.3).

### 4.1 Gas exchange in hydraulically-wide rivers

Field studies of gas exchange in wide rivers have suggested that k600 behaves differently in these rivers than in steeper and smaller rivers (Alin et al., 2011; Beaulieu et al., 2012; Raymond & Cole, 2001; Ulseth et al., 2019; Wang et al., 2021). While much work has focused on the small-stream side of the stream-to-river continuum, comparatively less work has been done in large rivers. Here, we focus on the larger, ‘smooth-channel’ end of the continuum, using a model for gas exchange that scales k600 by both ϵD and a shear Reynold’s number. Scaling k600 via a shear Reynold’s number is often done to parameterize breaking-wave gas exchange models in the open ocean (Brumer et al., 2017; Zhao et al., 2003; Zhao & Toba, 2001), though these models are specific to high wind speeds in open ocean. To our knowledge, Moog & Jirka (1999a)’s specific setup, which imposes a space-and-time varying, fractional area surface turbulence theory on the small-eddy model, has not been empirically validated in rivers until now (Figure 2 for hydraulically wide channels). Using our full dataset of k600, we also observe that this model breaks down when including non-hydraulically-wide rivers (as the theory would suggest). Future tests should also explore other shear Reynold’s scaling relations for gas exchange in rivers.

Crucially, we are not accounting for wind-driven gas exchange, which is suggested to play an important role in wide rivers because river surface area is sufficiently large that sheltering no longer limits the influence of wind-derived turbulence (Beaulieu et al., 2012; Raymond & Cole, 2001; Wang et al., 2021). None of the existing hydraulics-driven fluvial k600 models account for wind-driven gas exchange either. Additionally, under higher-wind scenarios the turbulent regime will switch from hydraulically-driven to wind-driven turbulence (Zappa et al., 2007) and the assumptions under-pinning BIKER will likely break down. BIKER’s outputs can therefore be interpreted as the ‘k600 under low-wind conditions’ when surface turbulence is dominated by hydraulics rather than wind. That said, BIKER’s flexible implementation is a good start towards eventually coupling hydraulics-driven gas exchange with wind-driven gas exchange under moderate-to-high wind scenarios.

### 4.2 Towards remote sensing of global spatiotemporal dynamics of k600 in large rivers

To date, the studies exploring the spatiotemporal dynamics of riverine gas exchange have arguably been held back by a lack of data. A few studies have investigated these dynamics, but they have been limited to individual rivers and/or limited field seasons (Hall et al., 2012; Sand-Jensen & Staehr, 2012). For example, Wallin et al. (2011) performed a preliminary analysis in northern Sweden relating cross-section specific temporal variability in gas exchange with channel slope, but they were limited to a mean of only 8 measurements per river in a single watershed. While these studies are a good start, they are insufficient for further developing process-level understandings of gas exchange at the global-scale.

Therefore, inferring k600 from SWOT data is an attractive option to address this problem of limited data. For reference, 95% of the SWOT-visible rivers globally (202,811) will have sufficient SWOT observations along the river to run BIKER at least once every 21 days, with most of the temperate and Arctic rivers having 3+ observations per 21-day cycle (Altenau et al., 2021). While BIKER will not directly measure k600, it does robustly infer temporal trends in k600 and reasonably infers the absolute magnitude of k600 (Figures 4-6). This many data will provide a novel dataset of k600 on a scale never before possible.

However, SWOT’s relatively coarse spatial resolution limits BIKER’s use to large rivers. This limitation means SWOT cannot see the vast majority of the global river network (which are too narrow for SWOT), though it is likely to observe much of its air/water interface at which gas exchange occurs (rivers wide enough for SWOT to observe). To confirm this hypothesis, we obtained the global estimates for SWOT-observable surface area and length (at mean annual streamflow- Altenau et al., 2021) and compared them to the most recent estimates of global river surface area and length (Liu et al., 2022- Table S4). We found that 42% of the global riverine surface area is SWOT-observable, while only 0.32% of the network length is SWOT-observable. While small streams in aggregate exert a significant influence on GHG emissions from river networks (Liu et al., 2022); Raymond et al. (2013)], BIKER will still be capable of inferring k600 for much of the global freshwater air/water interface.

With that said, there is a substantial range of BIKER performance across rivers (Figures 3, 4, S4, S5). These differences in performance are likely due to the representativeness of the priors, which makes sense as Section 2.3 and section 3.2.1 have effectively reduced k600 to a function of hydraulics that are nearly all directly measurable by SWOT. Any resulting bias in BIKER’s predictions is likely attributable to either bias in the priors used for the non-remotely sensed terms (Equation 12b) or in the k600 model itself (Equation 8, including the aforementioned wind errors). For SWOT discharge algorithms, authors have repeatedly shown that the ‘quality’ of prior river knowledge plays a large role in the success of discharge inversions (Andreadis et al., 2020; Brinkerhoff et al., 2020; Frasson et al., 2021; Tuozzolo et al., 2019) and our results here further corroborate this finding.

Furthermore, there is likely a minimum sufficient variability in SWOT observations that is necessary to strongly infer a k600 timeseries (Figure 6). ‘Hydraulic visibility’, i.e. the ability of a remote sensor to identify a hydrological response in the river (Garambois et al., 2017) is applicable here. If we apply hydraulic visibility to a sensor’s ability to identify temporal variations in k600, our results suggest a ‘minimally sufficient’ hydraulic variability in SWOT measurements is needed to improve upon the prior (in Figure 6b, suggested to be approximately >20% CV for this sample of rivers). This finding will be important once SWOT launches and BIKER is implemented at the global-scale. Although it is presently impossible to know whether SWOT will achieve ‘sufficient hydraulic visibility’ over its lifetime, recent similar work using the Landsat archive suggests that most rivers’ full flow regime will be observed by the SWOT satellite: Allen et al. (2020) found that the Landsat record observed 97% of streamflow percentiles in 90% of United States streamgauges. Landsat has an average temporal resolution of 16 days, which is approximately similar to the repeat cycle for SWOT. Further, SWOT will penetrate clouds and provide even more data on cloudy days (unlike Landsat’s optical sensor). With that said, the nominal lifespan of SWOT is only three years, within which certain streamflow magnitudes may not be experienced and reduce the chance that ‘sufficient hydraulic visibility’ is achieved.

Finally, because of its reliance on Manning’s equation and hydraulic geometry (section 3.2.1.), BIKER cannot invert overbank flow events, similar to many SWOT discharge algorithms. This concept is an important distinction that must be accounted for when BIKER is run on actual SWOT data, though future work should also look to couple floodplain flow laws with BIKER to capture gas exchange in seasonally-inundated floodplains.

### 4.3 Coupling BIKER with upscaling workflows

BIKER will likely be useful for informing annual upscaled carbon emissions estimates from river networks when coupled with CO2 data (Figure 7). This encouraging result has two main implications for future work.

First, it implies that BIKER will be useful when coupled with large-scale CO2water models, provided these models are accurate. The models would give time and space varying gas exchange. Liu et al. (2022) and Saccardi & Winnick (2021) each propose models that robustly predict reach-scale dissolved CO2 concentrations using two different approaches- machine learning for Liu et al. (2022) and process-based reactive transport modeling for Saccardi & Winnick (2021)- but both models yield CO2 estimates that would be spatially and temporally consistent with BIKER’s output. Our promising results suggest that BIKER could provide additional (and directly inferred) measurements of kCO2 to these models, thereby better informing model results through direct observations. This modeling would likely be accomplished via data assimilation which has proven useful in using remotely-sensed discharge to improve streamflow routing models (Feng et al., 2021; Ishitsuka et al., 2020), and of which the Saccardi & Winnick (2021) CO2 model takes a similar form.

Second, our experiments allow us to directly compare the influence of geomorphic assumptions on total carbon emission rates from river networks (Figure 7), as all other calculations and parameters were held constant across our four tested models (Text S6). Therefore, we highlight a potentially large source of uncertainty in current river CO2 upscaling estimates: the geomorphic models employed to scale river channel hydraulics with streamflow (Figure 7). In this case, the only difference between the three literature models and the observed estimate is the specific HG model employed to predict river depth and velocity (Text S6, Table S3), and yet the eventual carbon emissions estimates are quite different (Figure 7). Further, recall that the BIKER results in Figure 7 reflect a worst-case scenario (relatively uniformed priors), while the three in-situ methods represent best case scenarios (perfect streamflow records). We suggest future work should perform a formal sensitivity analysis for these HG parameters.

## 5 Conclusions

This proof-of-concept study verifies that BIKER can provide meaningful information on the spatiotemporal dynamics of gas exchange solely from SWOT data and functionally opens the door for a global-scale analysis of riverine gas exchange upon SWOT’s launch (and data collection). Although BIKER results are often biased in magnitude, they strongly capture the temporal dynamics of gas exchange velocity and will provide an unprecedented amount of new information on global riverine gas exchange that should be essential for better constraining existing river CO2 models.

## 6 Acknowledgements

C.B. Brinkerhoff was funded by a NASA Future Investigators in Earth and Space Science Fellowship (80NSSC21K1591). C.J. Gleason was supported by a NASA SWOT Science Team grant (80NSSC20K1141). We thank Renato Frasson, Michael Durand, Amber Ulseth, and Jake Beaulieu for generously making their data available for this study. We also thank the entire SWOT discharge algorithm working group for their decade-plus body of work which inspired this study.

## 7 Data availability statement

Datasets required for this research are available from Ulseth et al. (2019) (https://doi.org/10.1038/s41561-019-0324-8), Brinkerhoff et al. (2019) (https://doi.org/10.1029/2019GL084529), Frasson et al. (2021) (https://zenodo.org/record/3817817), Churchill et al. (1964) (https://pubs.usgs.gov/pp/0737/report.pdf), Owens et al. (1964) (https://pubs.usgs.gov/pp/0737/report.pdf), Beaulieu et al. (2012) (https://doi.org/10.1029/2011JG001794), and Durand et al. (2016) (https://doi.org/10.1002/2015WR018434). All code to generate results and figures is archived at https://zenodo.org/record/6914119.

## 8 Software availability statement

The BIKER algorithm remains in active development and is available at https://github.com/craigbrinkerhoff/BIKER. The specific version of BIKER used in this study is archived at https://zenodo.org/record/6914064.

## 9 Appendix A

Table A1: Variable description and notation for this study. Unit quantities are as follows: M for mass, L for length, T for time, P for pressure, and E for energy.

| Notation | Description | Calculation (if necessary) | Units |
| --- | --- | --- | --- |
| A | Channel cross-sectional area |  | L2 |
| A0 | Non-SWOT-observable cross-sectional area |  | L2 |
| Ap | Active zone fraction (Moog & Jirka 1999) | Ap∝Re*1/2(Moog & Jirka 1999) | dimensionless |
| CO2 | Carbon dioxide concentration |  | ML3 |
| CO2water | Water-side CO2 concentration |  | ML3 |
| CO2air | Atmospheric-side CO2 concentration |  | ML3 |
| dA | change in cross-sectional area | t′:Wt′≤Wt​Wt′δHet′ | L2 |
| Dm | Molecular diffusion coefficient |  | L2T |
| ϵ | Dissipation rate of near-surface turbulence |  | EM*T |
| ϵS | log-law-of-the-wall model for ϵ | U*3H | EM*T |
| ϵD | Form-drag model for ϵ | gSU | EM*T |
| FCO2 | CO2 evasion flux from river to atmosphere |  | ML2T |
| FCO2upscaled | Upscaling estimate of the global CO2 evasion flux from river to atmosphere |  | ML2T |
| g | gravitational acceleration | 9.8 | LT2 |
| H | Mean flow depth | AW | L |
| He | Water surface elevation |  | L |
| i | Cross-section discretization within a mass-conserved river reach |  |  |
| kz | gas exchange velocity for gas z |  | LT |
| k600 | gas exchange velocity normalized to Sc=600 | 600Sc−0.5k | LT |
| n | Manning’s roughness coefficient | Rh2/3S1/2U | TL3 |
| ρ | Density of water |  | ML1/3 |
| Q | River discharge | WHU | L3T |
| Rh | Hydraulic radius | AW2H+W | L |
| Re* | Shear Reynold’s number | U*Hv | dimensionless |
| S | River slope |  | dimensionless |
| Sc | Schmidt number | vDm |  |
| t | timestep discretization within river reach |  |  |
| Θ | Bayesian parameter set |  |  |
| U | Cross-sectional average velocity | QA | LT |
| U* | Shear velocity | gSRh | LT |
| μ | Viscosity |  | P*T |
| v | kinematic viscosity | μρ | L2T |
| W | Flow width |  | L |
| x | Bayesian data set |  |  |

Table A2: Bayesian parameter and hyperparameter notation for this study.

| Notation | Description |
| --- | --- |
| x | Joint data distribution for BIKER |
| Θ | Joint parameter distribution for BIKER |
| Θμ | Mean hyperparameter for BIKER prior distributions |
| Θσ | Standard deviation hyperparameter for BIKER prior distributions |
| Θγ | Upper bound hyperparameter for BIKER prior distributions |
| Θλ | Lower bound hyperparameter for BIKER prior distributions |
| σk600 | ‘Uncertainty’ hyperparameter for BIKER likelihood distribution |
| αn | Coefficient parameter distributions for k600 scaling relations, where n = independent variable number |
| β1 | Intercept parameter distribution for k600 scaling relations |
| σLM | Uncertainty parameter distribution for k600 scaling relations |

## References

Aho, K. S., & Raymond, P. A. (2019). Differential Response of Greenhouse Gas Evasion to Storms in Forested and Wetland Streams. Journal of Geophysical Research: Biogeosciences, 124(3), 649–662. https://doi.org/10.1029/2018JG004750

Aho, K. S., Fair, J. H., Hosen, J. D., Kyzivat, E. D., Logozzo, L. A., Rocher-Ros, G., et al. (2021). Distinct concentration-discharge dynamics in temperate streams and rivers: CO2 exhibits chemostasis while CH4 exhibits source limitation due to temperature control. Limnology and Oceanography, 66(10), 3656–3668. https://doi.org/10.1002/lno.11906

Alin, S. R., Rasera, M. de F. F. L., Salimon, C. I., Richey, J. E., Holtgrieve, G. W., Krusche, A. V., & Snidvongs, A. (2011). Physical controls on carbon dioxide transfer velocity and flux in low-gradient river systems and implications for regional carbon budgets. Journal of Geophysical Research: Biogeosciences, 116(G1). https://doi.org/https://doi.org/10.1029/2010JG001398

Allen, G. H., Yang, X., Gardner, J., Holliman, J., David, C. H., & Ross, M. (2020). Timing of Landsat Overpasses Effectively Captures Flow Conditions of Large Rivers. Remote Sensing, 12(9), 1510. https://doi.org/10.3390/rs12091510

Altenau, E. H., Pavelsky, T. M., Durand, M. T., Yang, X., Frasson, R. P. de M., & Bendezu, L. (2021). The Surface Water and Ocean Topography (SWOT) Mission River Database (SWORD): A Global River Network for Satellite Data Products. Water Resources Research, 57(7), e2021WR030054. https://doi.org/10.1029/2021WR030054

Andreadis, K. M., Brinkerhoff, C. B., & Gleason, C. J. (2020). Constraining the assimilation of SWOT observations with hydraulic geometry relations. Water Resources Research. https://doi.org/10.1029/2019WR026611

Appling, A. P., Hall, R. O., Yackulic, C. B., & Arroita, M. (2018). Overcoming Equifinality: Leveraging Long Time Series for Stream Metabolism Estimation. Journal of Geophysical Research: Biogeosciences, 123(2), 624–645. https://doi.org/https://doi.org/10.1002/2017JG004140

Beaulieu, J. J., Shuster, W. D., & Rebholz, J. A. (2012). Controls on gas transfer velocities in a large river. Journal of Geophysical Research: Biogeosciences, 117(G2). https://doi.org/10.1029/2011JG001794

Biancamaria, S., Lettenmaier, D. P., & Pavelsky, T. M. (2016). The SWOT Mission and Its Capabilities for Land Hydrology. In A. Cazenave, N. Champollion, J. Benveniste, & J. Chen (Eds.), Remote Sensing and Water Resources (pp. 117–147). Cham: Springer International Publishing. https://doi.org/10.1007/978-3-319-32449-4_6

Borges, A. V., Darchambeau, F., Teodoru, C. R., Marwick, T. R., Tamooh, F., Geeraert, N., et al. (2015). Globally significant greenhouse-gas emissions from African inland waters. Nature Geoscience, 8(8), 637–642. https://doi.org/10.1038/ngeo2486

Brinkerhoff, C. B., Gleason, C. J., & Ostendorf, D. W. (2019). Reconciling at-a-Station and at-Many-Stations Hydraulic Geometry Through River-Wide Geomorphology. Geophysical Research Letters, 46(16), 9637–9647. https://doi.org/10.1029/2019GL084529

Brinkerhoff, C. B., Gleason, C. J., Feng, D., & Lin, P. (2020). Constraining Remote River Discharge Estimation Using Reach-Scale Geomorphology. Water Resources Research, 56(11), e2020WR027949. https://doi.org/https://doi.org/10.1029/2020WR027949

Brinkerhoff, C. B., Raymond, P. A., Maavara, T., Ishitsuka, Y., Aho, K. S., & Gleason, C. J. (2021). Lake Morphometry and River Network Controls on Evasion of Terrestrially Sourced Headwater CO2. Geophysical Research Letters, 48(1), e2020GL090068. https://doi.org/https://doi.org/10.1029/2020GL090068

Brumer, S. E., Zappa, C. J., Blomquist, B. W., Fairall, C. W., Cifuentes-Lorenzen, A., Edson, J. B., et al. (2017). Wave-Related Reynolds Number Parameterizations of CO2 and DMS Transfer Velocities. Geophysical Research Letters, 44(19), 9865–9875. https://doi.org/10.1002/2017GL074979

Calderbank, P. H., & Moo-Young, M. B. (1961). The continuous phase heat and mass-transfer properties of dispersions. Chemical Engineering Science, 16(1), 39–54. https://doi.org/10.1016/0009-2509(61)87005-X

Churchill, M. A., Elmore, H. L., & Buckingham, R. A. (1964). The Prediction of Stream Reaeration Rates. In B. A. Southgate (Ed.), Advances in Water Pollution Research (pp. 89–136). Pergamon. https://doi.org/10.1016/B978-1-4832-8391-3.50015-4

Cole, J. J., Prairie, Y. T., Caraco, N. F., McDowell, W. H., Tranvik, L. J., Striegl, R. G., et al. (2007). Plumbing the Global Carbon Cycle: Integrating Inland Waters into the Terrestrial Carbon Budget. Ecosystems, 10(1), 172–185. https://doi.org/10.1007/s10021-006-9013-8

Crawford, J. T., Stanley, E. H., Dornblaser, M. M., & Striegl, R. G. (2017). CO2 time series patterns in contrasting headwater streams of North America. Aquatic Sciences, 79(3), 473–486. https://doi.org/10.1007/s00027-016-0511-2

Durand, M. T., Neal, J., Rodríguez, E., Andreadis, K. M., Smith, L. C., & Yoon, Y. (2014). Estimating reach-averaged discharge for the River Severn from measurements of river water surface elevation and slope. Journal of Hydrology, 511, 92–104. https://doi.org/10.1016/j.jhydrol.2013.12.050

Durand, M. T., Gleason, C. J., Garambois, P. A., Bjerklie, D., Smith, L. C., Roux, H., et al. (2016). An intercomparison of remote sensing river discharge estimation algorithms from measurements of river height, width, and slope. Water Resources Research, 52(6), 4527–4549. https://doi.org/10.1002/2015WR018434

Durand, M. T., Chen, C., Moraes Frasson, R. P. de, Pavelsky, T. M., Williams, B., Yang, X., & Fore, A. (2020). How will radar layover impact SWOT measurements of water surface elevation and slope, and estimates of river discharge? Remote Sensing of Environment, 247, 111883. https://doi.org/10.1016/j.rse.2020.111883

Feng, D., Gleason, C. J., Lin, P., Yang, X., Pan, M., & Ishitsuka, Y. (2021). Recent changes to Arctic river discharge. Nature Communications, 12(1), 6917. https://doi.org/10.1038/s41467-021-27228-1

Frasson, R. P. de M., Durand, M. T., Larnier, K., Gleason, C., Andreadis, K. M., Hagemann, M., et al. (2021). Exploring the factors controlling the error characteristics of the Surface Water and Ocean Topography mission discharge estimates. Water Resources Research, n/a(n/a), e2020WR028519. https://doi.org/https://doi.org/10.1029/2020WR028519

Garambois, P.-A., Calmant, S., Roux, H., Paris, A., Monnier, J., Finaud-Guyot, P., et al. (2017). Hydraulic visibility: Using satellite altimetry to parameterize a hydraulic model of an ungauged reach of a braided river. Hydrological Processes, 31(4), 756–767. https://doi.org/10.1002/hyp.11033

Gelman, A., Goodrich, B., Gabry, J., & Vehtari, A. (2019). R-squared for Bayesian Regression Models. The American Statistician, 73(3), 307–309. https://doi.org/10.1080/00031305.2018.1549100

Gleason, C. J. (2015). Hydraulic geometry of natural rivers: A review and future directions. Progress in Physical Geography: Earth and Environment, 39(3), 337–360. https://doi.org/10.1177/0309133314567584

Gleason, C. J., Garambois, P.-A., & Durand, M. T. (2017). Tracking River Flows from Space. Eos. https://doi.org/10.1029/2017EO078085

Gruber, N., Clement, D., Carter, B. R., Feely, R. A., Heuven, S. van, Hoppema, M., et al. (2019). The oceanic sink for anthropogenic CO2 from 1994 to 2007. Science, 363(6432), 1193–1199. https://doi.org/10.1126/science.aau5153

Hagemann, M. W., Gleason, C. J., & Durand, M. T. (2017). BAM: Bayesian AMHG-Manning Inference of Discharge Using Remotely Sensed Stream Width, Slope, and Height. Water Resources Research, 53(11), 9692–9707. https://doi.org/10.1002/2017WR021626

Hall, R. O., & Ulseth, A. J. (2020). Gas exchange in streams and rivers. WIREs Water, 7(1), e1391. https://doi.org/https://doi.org/10.1002/wat2.1391

Hall, R. O., Kennedy, T. A., & Rosi‐Marshall, E. J. (2012). Air–water oxygen exchange in a large whitewater river. Limnology and Oceanography: Fluids and Environments, 2(1), 1–11. https://doi.org/https://doi.org/10.1215/21573689-1572535

Hoff, P. D. (2009). A First Course in Bayesian Statistical Methods. New York: Springer.

Horgby, Å., Segatto, P. L., Bertuzzo, E., Lauerwald, R., Lehner, B., Ulseth, A. J., et al. (2019). Unexpected large evasion fluxes of carbon dioxide from turbulent streams draining the world’s mountains. Nature Communications, 10(1). https://doi.org/10.1038/s41467-019-12905-z

Ishitsuka, Y., Gleason, C. J., Hagemann, M. W., Beighley, E., Allen, G. H., Feng, D., et al. (2020). Combining optical remote sensing, McFLI discharge estimation, global hydrologic modelling, and data assimilation to improve daily discharge estimates across an entire large watershed. Water Resources Research, n/a(n/a). https://doi.org/https://doi.org/10.1029/2020WR027794

Katul, G., Mammarella, I., Grönholm, T., & Vesala, T. (2018). A Structure Function Model Recovers the Many Formulations for Air-Water Gas Transfer Velocity. Water Resources Research, 54(9), 5905–5920. https://doi.org/https://doi.org/10.1029/2018WR022731

Knoben, W. J. M., Freer, J. E., & Woods, R. A. (2019). Technical note: Inherent benchmark or not? Comparing Nash–Sutcliffe and Kling–Gupta efficiency scores. Hydrology and Earth System Sciences, 23(10), 4323–4331. https://doi.org/10.5194/hess-23-4323-2019

Lamont, J. C., & Scott, D. S. (1970). An Eddy Cell Model of Mass Transfer into the Surface of a Turbulent Liquid. AIChE Journal, 16(4), 513–519.

Lauerwald, R., Laruelle, G. G., Hartmann, J., Ciais, P., & Regnier, P. A. G. (2015). Spatial patterns in CO2 evasion from the global river network. Global Biogeochemical Cycles, 29(5), 534–554. https://doi.org/10.1002/2014GB004941

Leopold, L. B., & Maddock, T. (1953). The Hydraulic Geometry of Stream Channels and Some Physiographic Implications. U.S. Government Printing Office.

Liu, S., & Raymond, P. A. (2018). Hydrologic controls on pCO2 and CO2 efflux in US streams and rivers. Limnology and Oceanography Letters, 3(6), 428–435. https://doi.org/10.1002/lol2.10095

Liu, S., Kuhn, C., Amatulli, G., Aho, K., Butman, D. E., Allen, G. H., et al. (2022). The importance of hydrology in routing terrestrial carbon to the atmosphere via global streams and rivers. Proceedings of the National Academy of Sciences, 119(11), e2106322119. https://doi.org/10.1073/pnas.2106322119

Lorke, A., & Peeters, F. (2006). Toward a Unified Scaling Relation for Interfacial Fluxes. Journal of Physical Oceanography, 36(5), 955–961. https://doi.org/10.1175/JPO2903.1

Moog, D. B., & Jirka, G. H. (1999a). Air-Water Gas Transfer in Uniform Channel Flow. Journal of Hydraulic Engineering, 125(1), 3–10. https://doi.org/10.1061/(ASCE)0733-9429(1999)125:1(3)

Moog, D. B., & Jirka, G. H. (1999b). Stream Reaeration in Nonuniform Flow: Macroroughness Enhancement. Journal of Hydraulic Engineering, 125(1), 6.

Owens, M., Edwards, R. W., & Gibbs, J. W. (1964). Some reaeration studies in streams. Inter. J. Air Water Poll., 8, 469–486. Retrieved from https://ci.nii.ac.jp/naid/10025707421/

Pan, Y., Birdsey, R. A., Fang, J., Houghton, R., Kauppi, P. E., Kurz, W. A., et al. (2011). A Large and Persistent Carbon Sink in the World’s Forests. Science, 333(6045), 988–993. https://doi.org/10.1126/science.1201609

Peter, H., Singer, G. A., Preiler, C., Chifflard, P., Steniczka, G., & Battin, T. J. (2014). Scales and drivers of temporal pCO2 dynamics in an Alpine stream. Journal of Geophysical Research: Biogeosciences, 119(6), 1078–1091. https://doi.org/https://doi.org/10.1002/2013JG002552

Ran, L., Lu, X. X., & Liu, S. (2017). Dynamics of riverine CO2 in the Yangtze River fluvial network and their implications for carbon evasion. Biogeosciences, 14, 2183–2198.

Raymond, P. A., & Cole, J. J. (2001). Gas Exchange in Rivers and Estuaries: Choosing a Gas Transfer Velocity. Estuaries, 24(2), 312–317. https://doi.org/10.2307/1352954

Raymond, P. A., Bauer, J. E., & Cole, J. J. (2000). Atmospheric CO2 evasion, dissolved inorganic carbon production, and net heterotrophy in the York River estuary. Limnology and Oceanography, 45(8), 1707–1717. https://doi.org/https://doi.org/10.4319/lo.2000.45.8.1707

Raymond, P. A., Zappa, C. J., Butman, D., Bott, T. L., Potter, J., Mulholland, P., et al. (2012). Scaling the gas transfer velocity and hydraulic geometry in streams and small rivers. Limnology and Oceanography, 41–53. https://doi.org/10.1215/21573689-1597669@10.1002/(ISSN)1939-5590.MethaneVI

Raymond, P. A., Hartmann, J., Lauerwald, R., Sobek, S., McDonald, C., Hoover, M., et al. (2013). Global carbon dioxide emissions from inland waters. Nature, 503(7476), 355–359. https://doi.org/10.1038/nature12760

Rocher‐Ros, G., Sponseller, R. A., Lidberg, W., Mörth, C.-M., & Giesler, R. (2019). Landscape process domains drive patterns of CO2 evasion from river networks. Limnology and Oceanography Letters, 4(4), 87–95. https://doi.org/10.1002/lol2.10108

Saccardi, B., & Winnick, M. (2021). Improving Predictions of Stream CO2 Concentrations and Fluxes Using a Stream Network Model: A Case Study in the East River Watershed, CO, USA. Global Biogeochemical Cycles, 35(12), e2021GB006972. https://doi.org/10.1029/2021GB006972

Sand-Jensen, K., & Staehr, P. A. (2012). CO2 dynamics along Danish lowland streams: Water–air gradients, piston velocities and evasion rates. Biogeochemistry, 111(1), 615–628. https://doi.org/10.1007/s10533-011-9696-6

Talke, S. A., Horner-Devine, A. R., Chickadel, C. C., & Jessup, A. T. (2013). Turbulent kinetic energy and coherent structures in a tidal river. Journal of Geophysical Research: Oceans, 118(12), 6965–6981. https://doi.org/10.1002/2012JC008103

Tokoro, T., Kayanne, H., Watanabe, A., Nadaoka, K., Tamura, H., Nozaki, K., et al. (2008). High gas-transfer velocity in coastal regions with high energy-dissipation rates. Journal of Geophysical Research: Oceans, 113(C11). https://doi.org/10.1029/2007JC004528

Tuozzolo, S., Lind, G., Overstreet, B., Mangano, J., Fonstad, M., Hagemann, M., et al. (2019). Estimating River Discharge With Swath Altimetry: A Proof of Concept Using AirSWOT Observations. Geophysical Research Letters, 46(3), 1459–1466. https://doi.org/10.1029/2018GL080771

Ulseth, A. J., Hall, R. O., Boix Canadell, M., Madinger, H. L., Niayifar, A., & Battin, T. J. (2019). Distinct air–water gas exchange regimes in low- and high-energy streams. Nature Geoscience, 12(4), 259–263. https://doi.org/10.1038/s41561-019-0324-8

Vachon, D., Prairie, Y. T., & Cole, J. J. (2010). The relationship between near-surface turbulence and gas transfer velocity in freshwater systems and its implications for floating chamber measurements of gas exchange. Limnology and Oceanography, 55(4), 1723–1732. https://doi.org/10.4319/lo.2010.55.4.1723

Wallin, M. B., Öquist, M. G., Buffam, I., Billett, M. F., Nisell, J., & Bishop, K. H. (2011). Spatiotemporal variability of the gas transfer coefficient (KCO2) in boreal streams: Implications for large scale estimates of CO2 evasion. Global Biogeochemical Cycles, 25(3). https://doi.org/https://doi.org/10.1029/2010GB003975

Wang, J., Bombardelli, F. A., & Dong, X. (2021). Physically Based Scaling Models to Predict Gas Transfer Velocity in Streams and Rivers. Water Resources Research, 57(3), e2020WR028757. https://doi.org/https://doi.org/10.1029/2020WR028757

Zappa, C. J., Raymond, P. A., Terray, E. A., & McGillis, W. R. (2003). Variation in surface turbulence and the gas transfer velocity over a tidal cycle in a macro-tidal estuary. Estuaries, 26(6), 1401–1415. https://doi.org/10.1007/BF02803649

Zappa, C. J., McGillis, W. R., Raymond, P. A., Edson, J. B., Hintsa, E. J., Zemmelink, H. J., et al. (2007). Environmental turbulent mixing controls on air-water gas exchange in marine and aquatic systems. Geophysical Research Letters, 34(10). https://doi.org/10.1029/2006GL028790

Zhao, D., & Toba, Y. (2001). Dependence of Whitecap Coverage on Wind and Wind-Wave Properties. Journal of Oceanography, 57(5), 603–616. https://doi.org/10.1023/A:1021215904955

Zhao, D., Toba, Y., Suzuki, Y., & Komori, S. (2003). Effect of wind waves on air’sea gas exchange: Proposal of an overall CO2 transfer velocity formula as a function of breaking-wave parameter. Tellus B: Chemical and Physical Meteorology, 55(2), 478–487. https://doi.org/10.3402/tellusb.v55i2.16747